HOT WORK PERMIT

WorkSafeBC OHS Regulation Part 12

Sections 12.112, 12.114–12.126  |  B.C. Reg. 296/97  |  Working tool — must be reviewed by a qualified person before use. Not legal or regulatory advice.

1 PERMIT IDENTIFICATION

| Permit No. | Issue Date | Permit Valid Until (date / time) |
| --- | --- | --- |

| Company / Employer | Worksite / Project Name |
| --- | --- |

Exact Location of Hot Work

Description of Hot Work (welding / cutting / grinding / brazing / torch use — be specific)

Equipment to Be Used (oxy-fuel, arc welder, grinder, etc.)

2 FIRE & EXPLOSION HAZARD ASSESSMENT (s.12.116)

☐  Area cleared of combustible materials within a minimum 10 m (35 ft) radius, or combustibles covered with fire-resistant material

☐  Floor and wall openings, cracks, or gaps within radius covered to contain sparks

☐  Container, vessel, tank, pipe, or structure thoroughly cleaned before hot work, where applicable  (s.12.116)

☐  Where a flammable or explosive substance may be present, atmosphere tested by a qualified person and a written safe work procedure adopted before work begins, with retesting at intervals  (s.12.116)

☐  Fixed fire suppression / sprinkler system confirmed operational in the work area, where present

If this hot work takes place inside or on a confined space (tank, vessel, pit, vault, or similar enclosed structure), a separate Confined Space Entry Permit under OHSR Part 9 is also required — classification, atmospheric testing, standby person, and rescue plan requirements apply in addition to this permit.

3 EQUIPMENT & PPE (s.12.114–12.124)

☐  Welding / cutting equipment inspected for defects, leaks, oil, and grease before use  (s.12.119)

☐  Equipment used only for the gas type it is designed for  (s.12.118)

☐  Reverse-flow check valves / flashback arrestors installed on oxyfuel hoses  (s.12.120)

☐  Local exhaust ventilation in place to control welding / burning fume exposure  (s.12.114)

☐  Harmful coatings (lead, chromium, other toxic coatings) removed from base metal before work, where practicable  (s.12.115)

☐  Arc flash protection in place — screens, curtains, or partitions to protect nearby workers  (s.12.122)

☐  PPE worn: flame-resistant clothing, gloves, apron, eye / face protection, appropriate footwear  (s.12.123)

☐  Respiratory protection provided if effective ventilation is not practicable for short-duration or emergency work  (s.12.124)

☐  Work carried out in accordance with CSA Standard W117.2, Safety in Welding, Cutting and Allied Processes  (s.12.112)

HOT WORK PERMIT — Page 2

Attach to Page 1 — same permit

| Permit No. (ref Page 1) | Date |
| --- | --- |

4 FIRE WATCH & EXTINGUISHER (s.12.125–12.126)

☐  At least one fire extinguisher of a suitable type and capacity immediately available at the work location  (s.12.126)

☐  Fire watch assigned and present during all hot work

☐  Fire watch maintained for a minimum of 30 minutes after hot work is completed, and the area re-inspected before the watch stands down (industry best practice — not a numbered OHSR requirement; verify against site fire code and insurer requirements)

☐  Recently welded or flame-cut work marked “HOT” or effectively guarded to prevent contact by workers not involved in the work  (s.12.125)

| Fire Watch Name(s) | Fire Watch Sign-Off Time (after hot work + 30 min minimum) |
| --- | --- |

5 AUTHORIZED WORKERS

| Worker Name (print) | Signature | Time In | Time Out |
| --- | --- | --- | --- |

6 AUTHORIZATION & CLOSURE

Issuing Supervisor — by signing I confirm hazards have been assessed, controls in Sections 2–4 are in place, and all workers have been briefed:

| Signature — Issuing Supervisor | Printed Name | Date / Time |
| --- | --- | --- |

Permit Closed — sign to confirm hot work is complete, the fire watch period has elapsed, and the area has been re-inspected with no smoldering material or ignition risk remaining:

| Signature — Closing Supervisor | Printed Name | Date / Time |
| --- | --- | --- |

This template is a professional working tool. It must be reviewed by a qualified person prior to use and verified against the current WorkSafeBC OHS Regulation, applicable CSA standards, and site-specific fire code or insurance requirements. Not legal or regulatory advice.