| CONFINED SPACE ENTRY PERMIT | WorkSafeBC OHS Regulation Part 9 |
| --- | --- |
| Sections 9.1, 9.25, 9.29–9.36 ¦ B.C. Reg. 296/97 ¦ Working tool — must be reviewed by a qualified person before use. Not legal or regulatory advice. |  |

| 1 PERMIT IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
| Permit No. | Issue Date |  | Permit Valid Until (date / time) |
| Company / Employer |  | Worksite / Project Name |  |
| Confined Space Location / ID (address, tag no., drawing ref.) |  |  |  |
| Description of Space (type: tank / vault / manhole / pit / sump — be specific) |  |  |  |
| Work to be Performed Inside Space |  |  |  |

2 ATMOSPHERIC HAZARD CLASSIFICATION (s.9.1 / s.9.25)

LOW HAZARD: Clean respirable air: O2 ~20.9%, no measurable flammable gas/vapour, no contaminant >10% of exposure limit (s.9.1) MODERATE HAZARD: Not clean respirable air, but atmosphere still permits unaided escape if ventilation / respirator fails (s.9.25) HIGH HAZARD: Atmosphere may cause death, incapacitation, injury, acute illness, or prevent unaided escape if ventilation / respirator fails (s.9.25) Continuous monitoring required if flammable / explosive atmosphere >20% LEL could develop during entry (s.9.34). Ventilation: s.9.30–9.32.

| □ LOW HAZARD | □ MODERATE HAZARD | □ HIGH HAZARD |  |
| --- | --- | --- | --- |

Classification Rationale (document why this classification was assigned)

| 3 ISOLATION, LOCKOUT & PURGE VERIFICATION (s.9.30–9.32) |  |
| --- | --- |
| □ All energy sources isolated and locked out (LOTO per Part 10) | □ Pipelines: double block and bleed confirmed (bleed checked within 20 min of entry) |

| □ Space purged and ventilated prior to atmospheric testing | □ Mechanical ventilation running, flow confirmed (min 85 m³/h per worker — low hazard) |
| --- | --- |

| □ Supplied-air / SCBA required: YES / NO (circle — required for moderate & high hazard per s.9.29) |  |
| --- | --- |

Isolation / Lockout Details (equipment ID, lock numbers, person responsible)

| 4 REQUIRED PPE & EQUIPMENT |  |  |  |
| --- | --- | --- | --- |
| □ CSA Grade 1 safety footwear | □ Hard hat (CSA Z94.1) | □ Safety glasses / goggles | □ Face shield |

| □ SCBA (self-contained) | □ Supplied-air respirator | □ Emergency escape respirator | □ Air-purifying respirator (specify) |
| --- | --- | --- | --- |

| □ Safety harness & lifeline | □ Tripod / mechanical retrieval | □ Chemical-resistant gloves | □ High-visibility vest |
| --- | --- | --- | --- |

| □ Personal gas monitor | □ Continuous area gas monitor | □ Intrinsically safe lighting | □ Other (specify below) |
| --- | --- | --- | --- |

Respiratory / Supplied-Air Details (type, tank pressure, duration, cylinder No.)

Other PPE / Hazard-Specific Equipment (H2S monitor cal. date, escape pack location, etc.)

5 STANDBY PERSON REQUIREMENTS (s.9.35 / s.9.36)

LOW: 1 standby person; continuous means to summon rescue; checks on workers at least every 20 min MODERATE: 1+ standby persons at / near entrance; visual checks at least every 20 min; continuous means to summon rescue HIGH: 1+ standby persons AT entrance continuously attending; continuously monitoring workers; capable of immediate rescue; prevent lifeline entanglement HIGH hazard: emergency escape respirator must be within arm’s reach of or on each entrant at all times (s.9.36)

| Standby Person Name(s) | Standby Person Qualifications / Training |
| --- | --- |

| 6 RESCUE PLAN |  |
| --- | --- |
| Rescue Method (non-entry retrieval / entry rescue — specify) | Rescue Equipment On Site (tripod, SCBA, stretcher, AED, etc.) |
| Emergency Contact / Phone Number | Nearest Hospital / AED Location |

| 7 AUTHORIZED ENTRANTS — sign before entry, initial on exit |  |  |  |
| --- | --- | --- | --- |
| Entrant Name (print) | Signature (entry) | Time In | Time Out / Initials |

8 AUTHORIZATION & CLOSURE

Issuing Supervisor — by signing I confirm conditions are safe, controls in place, and all entrants briefed:

| Signature — Issuing Supervisor | Printed Name | Date / Time |
| --- | --- | --- |

Permit Cancelled / Space Vacated — sign to formally close permit and record time all workers exited:

| Signature — Closing Supervisor | Printed Name | Date / Time |
| --- | --- | --- |

CONFINED SPACE ENTRY PERMIT — Page 2: Atmospheric Monitoring Continuation Attach to Page 1 — same permit

| Permit No. (ref Page 1) | Date |
| --- | --- |

VENTILATION REQUIREMENTS (s.9.30–9.32)

Low hazard: Minimum 85 m³/h (50 cfm) of clean respirable air per worker via mechanical ventilation (unless all low-hazard exceptions met). Moderate hazard: Continuous mechanical ventilation supplying clean respirable air required at all times (s.9.31). High hazard: Continuous mechanical ventilation required; natural ventilation NOT permitted. Supplied-air respirator or SCBA required when ventilation is insufficient (s.9.29 / s.9.32).

| EXTENDED ATMOSPHERIC MONITORING LOG (s.9.34 — retest if all workers absent >20 min; continuous monitor if >20% LEL possible) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Time | O2 (%) ≥19.5% | LEL (%) <20% | H2S (ppm) ≤10 ppm (ceiling) | CO (ppm) <25 ppm TWA | Other Gas (ID & ppm) | Equipment & Cal. Date | Tester Name & Signature |

| ENTRANTS — CONTINUATION (additional entries — attach to Page 1) |  |  |  |
| --- | --- | --- | --- |
| Entrant Name (print) | Signature (entry) | Time In | Time Out / Initials |

SUPERVISOR NOTES / DEVIATIONS / PERMIT AMENDMENTS

Notes (record any condition changes, deviations, additional controls, or permit amendments during work):

PERMIT AMENDMENT SIGNATURES (if conditions change, re-evaluate and re-authorize)

Amendment 1 — revised conditions assessed, re-briefing complete, permit valid to continue:

| Signature — Amending Supervisor | Printed Name | Date / Time |
| --- | --- | --- |

Amendment 2:

| Signature — Amending Supervisor | Printed Name | Date / Time |
| --- | --- | --- |